design and development of hybrid self checkout system with barcode scanner and camera

muhammad irfan fahmi bin samsul kamar

UNIVERSITI TEKNIKAL MALAYSIA MELAKA

design and development of hybrid self checkout system with barcode scanner and camera

muhammad irfan fahmi bin samsul kamar

This report is submitted in partial fulfilment of the requirements for the degree of Bachelor of Computer Engineering with Honours

Faculty of Electronics and Computer Technology and Engineering

Universiti Teknikal Malaysia Melaka

2026

UNIVERSITI TEKNIKAL MALAYSIA MELAKA

FAKULTI TEKNOLOGI DAN KEJURUTERAAN ELEKTRONIK DAN KOMPUTER

BORANG PENGESAHAN STATUS LAPORAN

PROJEK SARJANA MUDA II

Tajuk Projek	:  	Sila tuliskan tajuk projek anda disini

Sesi Pengajian       	: 	2010/2011

Saya nama penuh dalam huruf besar mengaku membenarkan laporan Projek Sarjana Muda ini disimpan di Perpustakaan dengan syarat-syarat kegunaan seperti berikut:

- Laporan adalah hakmilik Universiti Teknikal Malaysia Melaka.

- Perpustakaan dibenarkan membuat salinan untuk tujuan pengajian sahaja.

- Perpustakaan dibenarkan membuat salinan laporan ini sebagai bahan pertukaran antara institusi pengajian tinggi.

- Sila tandakan ():

|  | SULIT* | (Mengandungi maklumat yang berdarjah keselamatan atau kepentingan Malaysia seperti yang termaktub di dalam AKTA RAHSIA RASMI 1972) |  |
| --- | --- | --- | --- |
|  | TERHAD* | (Mengandungi maklumat terhad yang telah ditentukan oleh organisasi/badan di mana penyelidikan dijalankan. |  |
|  | TIDAK TERHAD |  |  |
|  |  |  | Disahkan oleh: |
| (TANDATANGAN PENULIS) |  |  | (COP DAN TANDATANGAN PENYELIA) |
| Alamat Tetap: Sila taip alamat penuh |  |  |  |
| Tarikh : 01 Januari 2010 |  |  | Tarikh : 01 Januari 2010 |

*CATATAN: Jika laporan ini SULIT atau TERHAD, sila lampirkan surat daripada pihak berkuasa/organisasi berkenaan dengan menyatakan sekali tempoh laporan ini perlu dikelaskan sebagai SULIT atau TERHAD.

Declaration

I declare that this report entitled “PSM Full Title” is the result of my own work except for quotes as cited in the references.

Signature	:	…………………………………

Author	: 	…………………………………

Date	:	…………………………………

Approval

I hereby declare that I have read this thesis and in my opinion, this thesis is sufficient in terms of scope and quality for the award of Bachelor of Electronic Engineering with Honours

Signature			:	…………………………………

Supervisor Name		: 	…………………………………

Date			:	…………………………………

Dedication

Click here to enter text.

Abstract

Radio Frequency Identification (RFID) is now the most popular technology for automating stores, however it has certain unique technical problems when used in food stores. When standard UHF RFID systems interact with products that have a lot of liquid or metal packaging, the electromagnetic waves can be absorbed and reflected, which can cause signal deterioration and read mistakes. In different retail settings, this physical limitation might make systems less reliable and inventory less accurate. This project aims to overcome these restrictions by developing an optimised Hybrid Smart Checkout System utilising the Raspberry Pi 5. The main goal is to make automated checkout more reliable by adding YOLOv8-based computer vision and barcode integration to older approaches. This method works well to reduce the signal interference problems that come with RFID. It also makes it much easier to detect "hard-to-read" commodities like bottled water and canned goods. The system has a proprietary multi-threaded software design that works well with the Raspberry Pi 5's quad-core processor. The system achieves low-latency inference suited for real-time operation by using asynchronous video capture, temporal frame sampling, and real-time heat monitoring. The system also makes it easier for users to access by adding a Voice-Responsive Assistant and a Web-Based Monitoring Interface. This solves the problem of not being able to supervise remotely in current commercial systems. Comparative study demonstrates that this prototype significantly mitigates the scanning faults linked to RFID while providing a more economical implementation. The end result is a very dependable and useful technology that makes it easy to scan regular grocery products, no matter what they are composed of.

Abstrak

Click here to enter text.

Acknowledgements

(Please delete this part): Most research reports, dissertations or theses have their subsection to convey appreciation to those who have been involved in the study.

Use Ack title for acknowledgement heading and for style TOC1.

Table of Contents

Declaration	i

Approval	i

Dedication	i

Abstract	i

Abstrak	iii

Acknowledgements	iv

Table of Contents	v

List of Figures	vii

List of Tables	viii

List of Symbols and Abbreviations	ix

List of Appendices	x

CHAPTER 1 INTRODUCTION	1

1.1	Project Background	1

1.2	Problem Statement	3

1.3	Project Objectives	3

1.4	Scope of Study	4

CHAPTER 2 LITERATURE REVIEW	5

2.1	Physics of RFID Failure in Grocery Environments	5

2.2	Computer Vision as an Alternative Identification Method	6

2.3	Multithreaded Optimisation for Edge AI	7

2.4	Cost Comparison of Self-Checkout Technologies	7

2.5	Research Gap Summary	9

CHAPTER 3 METHODOLOGY	11

3.1	System Architecture	11

CHAPTER 4 RESULTS AND DISCUSSION	13

CHAPTER 5 CONCLUSION AND FUTURE WORKS	14

REFERENCES	15

LIST OF PUBLICATIONS AND PAPERS PRESENTED	17

APPENDICES	18

List of Figures

(Please delete this part): This list contains the titles of figures, together with their page numbers, which are listed in the text. For e.g., figures in Chapter 3 are numbered sequentially: Figure 3.1, Figure 3.2.

For title of list tables use other title and TOC1 for style.

Figure 3.1: Example	6

List of Tables

(Please delete this part): This list contains the titles of tables, together with their page numbers, which are listed in the text. The numbering system is according to chapter, for e.g.: tables in Chapter 3 are numbered sequentially: Table 3.1, Table 3.2.

Table 2.1: Example	4

List of Symbols and Abbreviations

For examples:

| CC | : | Central canal |
| --- | --- | --- |
| DAB | : | 3,3’-diaminobenzidine |
| HRP | : | Horseradish peroxidase |
| MS222 | : | Tricaine methanesulfonate |
|  | : |  |
|  | : |  |

List of Appendices

| Appendix A: Example ……………………………………………………………... | 12 |
| --- | --- |

# INTRODUCTION

## Project Background

The retail industry is experiencing a paradigm shift toward Retail 4.0, where cyber-physical systems and edge intelligence are integrated into physical stores to improve operational efficiency and customer experience [1]. One of the most widely adopted technologies under this paradigm is the Self-Checkout System (SCO), which aims to reduce labour costs and increase checkout throughput [2].

Conventional SCOs rely primarily on manual barcode scanning, which introduces significant checkout friction. Customers often struggle with product orientation, barcode occlusion, or repeated scan failures, leading to delays and increased staff intervention [2]. These usability issues reduce the intended efficiency gains of self-checkout systems, especially in high-traffic grocery environments.

To overcome barcode limitations, Radio Frequency Identification (RFID) has been explored as a successor technology. While RFID has demonstrated success in apparel retail, its deployment in grocery stores remains problematic. Violino et al. [3] demonstrated that liquid-filled products significantly attenuate Ultra-High Frequency (UHF) signals due to dielectric absorption, while Wang and Chen [4] showed that metallic packaging reflects RF energy, causing antenna detuning and read blind spots. These physics-based constraints reduce RFID read rates and make the technology unreliable for Fast-Moving Consumer Goods (FMCG).

Consequently, there is a growing engineering need for a hybrid checkout solution that bypasses electromagnetic limitations. This project proposes a Smart Hybrid Self-Checkout System based on the Raspberry Pi 5, integrating YOLOv8-based computer vision with traditional barcode scanning. Unlike RFID, computer vision relies on optical features and is therefore immune to material composition issues [5].

To address the computational limitations of edge devices, the system incorporates a multithreaded video acquisition architecture, decoupling camera input from inference processing. This design improves frame stability and real-time responsiveness, as recommended in edge AI optimisation studies [10].

## Problem Statement

- RFID reliability drops in grocery environments due to signal interference from liquids and metal packaging, causing frequent missed reads.

- Current self-checkout systems lack remote monitoring and intuitive user interaction, offering limited voice support and no accessible web-based oversight.

## Project Objectives

- To develop a hybrid system combining barcode and YOLOv8-based computer vision to overcome RFID limitations with liquid/metal-packed items.

- Implement voice-responsive interaction and a web-based monitoring interface for real-time remote supervision and user assistance.

## Scope of Study

Hardware Platform

The system is implemented on a Raspberry Pi 5 (8 GB), selected for its quad-core Cortex-A76 CPU and suitability for edge AI applications [8].

Software Framework

The system is developed using Python 3 and Ultralytics YOLOv8 for object detection. A custom multithreaded pipeline is implemented to optimise video throughput. Flask is used for the web-based monitoring interface, while SQLite handles local transaction logging.

Data Domain

A custom dataset consisting of Malaysian grocery products (e.g., instant noodles, bread, canned goods) is used to demonstrate applicability in local SME retail environments.

# LITERATURE REVIEW

## Physics of RFID Failure in Grocery Environments

RFID performance in grocery retail is constrained by fundamental electromagnetic principles. Liquid products contain water, a polar molecule with a high dielectric constant that absorbs UHF signals, resulting in attenuation levels of up to −20 dB [3]. This absorption prevents sufficient energy from reaching RFID tags, rendering them unreadable.

Metallic packaging introduces a different challenge. Conductive surfaces reflect RF waves and alter antenna impedance, shifting the tag’s resonant frequency away from the reader’s operating band. Wang and Chen [4] identified this detuning effect as a primary cause of blind spots in metal-packaged goods. These limitations make RFID unsuitable as a standalone checkout solution for FMCG products.

## Computer Vision as an Alternative Identification Method

Computer vision (CV) systems identify products using visual features rather than electromagnetic signals, making them inherently immune to material composition issues. Recent advances in deep learning have enabled real-time object detection using lightweight models such as YOLO [9].

Tan et al. [5] demonstrated that YOLOv8 achieves improved mean Average Precision (mAP) while reducing computational overhead compared to earlier YOLO versions. This makes YOLOv8 suitable for deployment on resource-constrained edge devices. Al-Masni et al. [9] further confirmed the feasibility of YOLO-based detection in smart retail environments.

Benchmarking studies by Boddu [8] showed that the Raspberry Pi 5 can achieve real-time inference when lightweight models are used, validating its suitability for edge-based checkout systems.

## Multithreaded Optimisation for Edge AI

Edge AI systems often suffer from latency when video acquisition and inference are executed sequentially. Zhang [10] highlighted that blocking I/O operations significantly reduce system responsiveness and frame rate stability.

Multithreaded architectures mitigate this issue by separating video capture, inference, and I/O tasks into parallel execution threads. Yuen [21] demonstrated that such architectures reduce CPU idle time and improve power efficiency in IoT systems. This approach is critical for maintaining real-time performance on single-board computers.

## Cost Comparison of Self-Checkout Technologies

A major barrier to advanced self-checkout adoption in SMEs is cost. RFID-based systems require expensive readers and incur recurring tag costs, which significantly increase total ownership cost [12]. In contrast, vision-based systems leverage low-cost cameras and open-source software, reducing both capital and operational expenses.

| No. | Aspect | Barcode-Only SCO | RFID-Based SCO | Proposed Hybrid System |
| --- | --- | --- | --- | --- |
| 1 | Main Processor | Industrial PC | PLC / Controller | Raspberry Pi 5 |
| 2 | Identification Hardware | Barcode Scanner | RFID Reader | Barcode + Webcam |
| 3 | Initial Hardware Cost | Medium | Very High | Low |
| 4 | Recurring Cost | None | RFID Tags Required | None |
| 5 | Liquid Item Reliability | High | Poor [3] | High |
| 6 | Metal Item Reliability | High | Poor [4] | High |
| 7 | Installation Complexity | Low | High | Low |
| 8 | Maintenance Cost | Medium | High | Low |
| 9 | Loss Prevention | Low | Medium | High |
| 10 | Accessibility Support | Limited | Limited | Voice-Enabled |
| 11 | SME Affordability | Moderate | Poor | Excellent |
| 12 | Scalability Cost | Medium | Very High | Low |
| 13 | Power Consumption | Medium | High | Low |
| 14 | Software Cost | Proprietary | Proprietary | Open-Source |
| 15 | Upgrade Cost | High | Very High | Low |
| 16 | Edge AI Capability | None | None | Available |
| 17 | Camera Requirement | Optional | None | Single Camera |
| 18 | Monitoring Capability | Local Only | Local Only | Web-Based |
| 19 | Total Initial Cost | ~RM 2,500 | >RM 5,000 | <RM 600 |
| 20 | Long-Term Cost | Medium | Very High | Low |

Table 2.1: Cost Comparison of Self-Checkout Systems

## Research Gap Summary

Although extensive research exists on RFID-based checkout systems and vision-based retail applications, most studies address these domains independently. RFID studies focus on signal optimisation but fail to overcome fundamental physics limitations [3], [4]. Vision-only systems struggle with similar-looking products and lack redundancy [5]. Additionally, accessibility and edge optimisation are often overlooked [14], [25].

| No. | Author (Year) | Focus | Identified Gap | Project Contribution |
| --- | --- | --- | --- | --- |
| 1 | Violino et al. (2021) [3] | RFID & Liquids | Signal attenuation | Vision-based detection |
| 2 | Wang & Chen (2022) [4] | RFID & Metals | Detuning blind spots | Optical recognition |
| 3 | Truman (2022) [2] | SCO UX | Checkout friction | Voice guidance |
| 4 | Derbek (2022) [1] | Retail 4.0 | High automation cost | Low-cost edge system |
| 5 | Gupta (2021) [18] | Load-Cell Carts | Weight errors | Vision identity check |
| 6 | Rahim (2021) [19] | Arduino Billing | Low processing power | RPi 5 AI platform |
| 7 | Yuen (2021) [21] | IoT Energy | High power usage | Frame skipping |
| 8 | Al-Ali (2021) [22] | Cloud Retail | Latency dependency | Fully local processing |
| 9 | Lee & Park (2021) [25] | Elderly UX | Touchscreen anxiety | Voice interface |
| 10 | Mo & Zhang (2021) [33] | RFID Placement | Orientation sensitivity | Omni-angle vision |
| 11 | Tan et al. (2024) [5] | Vision Retail | Misclassification | Hybrid verification |
| 12 | Al-Masni et al. (2023) [9] | YOLO Systems | Heavy models | YOLOv8 Nano |
| 13 | Li et al. (2023) [13] | Fruit Vision | Raw produce only | Packaged goods |
| 14 | Kumar (2023) [12] | Retail AI | GPU dependence | CPU-only AI |
| 15 | Zhang (2023) [10] | Edge Latency | Blocking I/O | Multithreading |
| 16 | Sawant (2022) [20] | Barcode Security | Fake scans | Vision confirmation |
| 17 | Williams (2022) [14] | Accessibility | Visual-only UI | Audio feedback |
| 18 | Boddu (2025) [8] | RPi 5 AI | No retail use case | Checkout deployment |
| 19 | Truman (2022) [2] | User Errors | High error rate | Guided workflow |
| 20 | Derbek (2022) [1] | Retail Cost | SME exclusion | Affordable design |

# METHODOLOGY

## System Architecture

- The proposed system adopts a multithreaded architecture to ensure real-time performance on edge hardware. The architecture consists of three parallel threads:

Video Acquisition Thread – continuously captures frames to eliminate blocking I/O latency.

Inference Thread – performs YOLOv8 inference with frame skipping to stabilise FPS.

Asynchronous I/O Thread – handles voice output and web-based monitoring independently.

This architecture follows best practices for low-latency edge AI systems [10].

## Hardware Implementation

The processing unit is a Raspberry Pi 5 (8 GB). Visual input is provided by a 1080p webcam, downscaled to 640 × 480 to balance detection accuracy and inference latency, as recommended by Li et al. [13].

## Hybrid Detection Logic

To overcome RFID limitations, a priority-based hybrid detection logic is implemented:

- Barcode input is processed when available.

- If no barcode is detected, YOLOv8 inference is performed.

- Items are accepted when visual confidence exceeds 0.8.

This logic ensures reliable detection regardless of packaging material, directly addressing the limitations reported in RFID-based studies [3], [4].

# RESULTS AND DISCUSSION

For making numbered lists use “numbered lists style “and for bulleted list use “Bullet list” style. For both of them you will have 1 cm indention.

Numbered list example

Bullet list example

For explanation under bullet or numbered list use “indent” style which start a paragraph with 1.4 cm indention.

Do not change on Normal or text style because this is the base style for others so any change will affect other styles as well.

# CONCLUSION AND FUTURE WORKS

For making numbered lists use “numbered lists style “and for bulleted list use “Bullet list” style. For both of them you will have 1 cm indention.

Numbered list example

Bullet list example

For explanation under bullet or numbered list use “indent” style which start a paragraph with 1.4 cm indention.

Do not change on Normal or text style because this is the base style for others so any change will affect other styles as well.

REFERENCES

(Please delete this part): All works or studies referred to in the research report/dissertation/thesis in the form of quotations or citations must be included in the references. The references should be written consistently in the IEEE format or in another format approved by the faculty.

In order to insert new citation, go to REFERENCE>INSERT CITATION>GO TO MENDELEY then click CITE.

Use Reference style.

Examples: Author X discovered that this PSM report is simple [1]. In addition B. Wang et al. studied about temperature [2].

Conference paper:

[Number] Author, “Title of paper,” in Abbreviated Name of Conf., location of conference (optional), Month and day(s) if provided, year, pp. xxx-xxx.

[1]	A. Jaafar, N. Soin, and S. W. M. Hatta, “An Educational FPGA Design Process Flow using Xilinx ISE 13.3 Project Navigator for Students,” 2017 IEEE 13th Int. Colloq. Signal Process. its Appl., no. March, pp. 7–12, 2017.

[2]	B. Wang, R. Liang, L. Liu, J. Wang, and J. Xu, “Temperature dependence of The Hole Mobility in Indacenodithiophene-benzothiadiazole OTFTs,” 2017 IEEE 24th Int. Symp. Phys. Fail. Anal. Integr. Circuits (IPFA), Chengdu, pp. 1–3, 2017.

Journal article

[Number] First Name Initial(s) Last Name. “Page Title.” Website Title. Web Address (accessed Month Day Year).

[3] 	R. W. Kuhlenbeck and V. Kanrev, “Thermal Analysis in Simulation R4: Sensors processing,” NASA Thermal Eng., vol. 63, pp. 23-41, Nov. 2000.

LIST OF PUBLICATIONS AND PAPERS PRESENTED

Published works as well as papers presented at conferences, seminars, symposiums etc pertaining to the research topic of the research report/ dissertation/ thesis are suggested be included in this section. The first page of the article may also be appended as reference.

APPENDICES

(Please delete this part): Appendices consist of additional illustration of data sources, raw data and quoted citations which are too long to be placed in the text. The appendix supports the written text of the research report/dissertation/thesis. Research instruments such as questionnaires, maps or computer programmes are parts of appendix too.

Appendices can be divided into Appendix A, B, C.

This page is optional; if you do not have any appendices, delete the entire page.